CS 655: Computer Networks

Fall 2020

Sample Midterm Examination

Please write clearly and neatly. Be precise in your answers – do not just re-iterate what you know about the topic. Clearly state any assumptions you make. The exam has 4questions over 5pages. Answer all questions.

Problem 1 (Error Control)

A 3000-km long, 1 Mbps link is used to transmit 1000-bitdata packets using the Selective Repeat protocol. If the speed of light in this link is 2 x10^5km/second, what is the minimum number of bits the sequence numbers should be? Assume no flow control, and negligible transmission and processing times for acknowledgments. Take 1M = 1000,000.

Sender window size:

SWS = RTT x bandwidth = 2 x 3000 km / ( 2 x 10^5 km/s) x 1 Mbps = 30000 bits

Number of packets: 30000 bits / 1000 bits = 30 packets

We need 30 x 2 = 60 packets or sequence numbers to make Selective Repeat protocol work well.

Thus, we need 6 bits of sequence numbers.

Problem 2(Protocol Specification)

Problem 3 (HTTP Performance)

Propagation delay = 300ms / 2 = 150ms

2 x RTP + O / R

= 150ms + 150ms + 150ms + 150ms + (100 Kb / 100 Mbps) = 601ms to get the HTML file.

1 x RTP + O / R x 10

= 150ms + 150ms + (100 Kb / 100 Mbps) x 10 = 310ms to get the jpg files.

Response time = 2 x RTP + 1 x RTP + O / R x 10 = 3 x RTP + O / R x 11 = 911ms

Problem 4 (Caching)

- 首先，host跟local HTTP cache发起get请求，

local http cache向local DNS server发起DNS查询并得到返回，

local http cache发现缓存没有，于是向www.b.com发起TCP connection, 1.5个RTT，

文件传输时间为发送时延+传播时延+处理时延+排队时延，

发送时延取决于链路中的短板，1Gb / 1Mbps = 1000 s

传输时延为0.5个RTT,

Response time = 2RTT + 1000s = 1002 s

- 1s 直接从local http cache获取。

- Local http cache向local DNS server发起DNS query,

local DNS server向root DNS server发起请求再返回，1个RTT,

local DNS server向com DNS server发起请求再返回，1个RTT,

local DNS server向Authoritative DNS server发起请求再返回，1个RTT,

local DNS server把获知的IP address归还给local http cache.